| Continuous Assessment Cover Sheet |  |
| --- | --- |
| Student Name: Hanna Bokariuk Maryna Hordiienko Nikita Smiichyk | Student Number: D00283065 D00283071 D00283070 |

| Programme: Software Development | Stage: 3 | Complete Student Checklist: Re-read brief o References and Bibliography o Proofread o |
| --- | --- | --- |
| Module: Universal Design Project, IoT |  |  |
| Due Date: 11/10/26 | No. Pages: - |  |
| Lecturer’s Name: Enda Finn, Frank Keenan, Bride Mallon, John Loane |  |  |
| Assignment No. and/or Description/Topic: G-CA1 |  | Mode of Submission: Softcopy o Hardcopy o |

DECLARATION: I declare that: - This work is entirely my own, and no part of it has been copied from any other person’s words or ideas, except as specifically acknowledged through the use of inverted commas and in-text references; - No part of this assignment has been written for me by any other person except where such collaboration has been authorised by the lecturer concerned; - I understand that I am bound by DkIT Academic Integrity Policy. I understand that I may be penalised if I have violated the policy in any way; - This assignment has not been submitted for any other module at DkIT or any other institution, unless authorised by the relevant Lecturer(s); - I have read and abided by all of the requirements set down for this assignment. Signature ………………………………………………………….. Date …11/10/26…………

Lecturer’s  Comments:

Provisional Mark :   	     Lecturers Signature :  			_________   Date: _________

Work submitted late will be subject to penalties in accordance with the DkIT Continuous Assessment Policy

# Smart Box TempSafe

IoT Development G-CA1 Design Document

Class group: SD3a

Nikita Smiichyk | Hanna Bokariuk | Maryna Hordiienko

Repository: https://github.com/nikitsya/Smart-Box

# Contents

Main design

Appendix A Hardware and wiring

Appendix B Database schema and queries

Appendix C UI mock-ups and interactions

Appendix D Persona

Appendix E Functional requirements

Appendix F Initial test plan

Appendix G Submission review

# Main design

## 1. Purpose, users and scope

Smart Box (also called TempSafe) monitors temperature inside one prototype medication box. The intended user is an Advanced Paramedic who needs clear warnings and a trustworthy history with minimal interaction.

| User story | Acceptance evidence |
| --- | --- |
| As an assigned paramedic, I want to see current temperature and its age so I can distinguish a current warning from old data. | TP-03, TP-11 and UX-05 in the initial test plan. |
| As a paramedic, I want local warnings during an internet outage so I can recognise a problem near the box. | TP-08, TP-09 and TP-14. |
| As an assigned user, I want a graph and readable table so I can review earlier temperature changes. | TP-13 and UX-04. |
| As an assigned user, I want clear phone warnings so I can identify if box needs attention. | TP-18 to TP-20 and UX-06. |

## 2. Hardware, power and procurement

Use the team's existing Raspberry Pi 400, Adafruit SCD-41 breakout, microSD card, breadboard, jumper wires and one single-colour LED with a current-limiting resistor. Connect sensor VIN to physical pin 1 (3.3 V), GND to pin 6, SDA to pin 3 and SCL to pin 5. Proposed LED connection: physical pin 11 (BCM GPIO17), 1 kΩ series resistor, LED anode; LED cathode to physical pin 9 (ground). Verify polarity and actual LED ratings before applying power. Never connect an LED directly to a GPIO pin. The buzzer is optional and needs a circuit selected from its actual current rating; its GPIO connection is not yet approved.

The Raspberry Pi 400 will be powered using a compatible 5 V / 3 A USB-C mains power supply. For the G-CA1 bench prototype, the Raspberry Pi will remain outside the monitored box, with the SCD-41 sensor positioned inside the box and connected through a protected cable opening. This helps reduce the effect of heat generated by the Raspberry Pi on the temperature measurement. The Raspberry Pi will connect to the internet using Wi-Fi. As the current prototype is mains-powered and is not designed for standalone mobile operation, a power bank and GSM module are not required. Portable power and mobile connectivity may be considered as future work.

Before submission, Hanna inventories owned parts and identifies missing LED/resistor, enclosure and cables. Wi-Fi connects the Pi to the router; MQTT uses that internet connection rather than a direct laptop cable. No third-party data API is used.

Manufacturer references: Adafruit breakout pinouts [source], Raspberry Pi GPIO [source], Sensirion SCD41 and linked datasheet [source]. The SCD41 primarily measures CO₂ using photoacoustic sensing and contains integrated temperature/humidity sensing. Only temperature is retained in this project. The Sensirion datasheet, version 1.5, sections 3.5 and 3.6 [source] describes a five-second update interval in standard periodic mode. The proposed application reads a data-ready temperature every 30 seconds; sensor update frequency and stored-sample frequency are different. Validate placement/self-heating and any offset against a reference thermometer before interpreting alerts.

## 3. Architecture and secure pub-sub communication

Sensor and LED → local SQLite queue → MQTT/TLS broker → ingestion worker → MySQL. The HTTPS API serves authorised users; acknowledgements return through the broker, and the worker queues phone notifications.

A cloud virtual machine hosts Mosquitto, an ingestion worker, FastAPI and MySQL for the prototype. Hosting provider, region, domain and account ownership require team selection; no deployment is claimed. MySQL listens only on the private/local interface. Restrict public access to HTTPS and authenticated MQTT/TLS; administration uses restricted SSH access.

| Topic | Publisher | Subscriber | Policy |
| --- | --- | --- | --- |
| smartbox/v1/boxes/{box_id}/samples | Assigned device | Ingestion worker | QoS 1, non-retained; historical events. |
| smartbox/v1/boxes/{box_id}/heartbeat | Assigned device | Ingestion worker | QoS 1, non-retained; live contact only. |
| smartbox/v1/boxes/{box_id}/challenge | Ingestion worker | Assigned device | QoS 1, non-retained; expiring single-use heartbeat nonce. |
| smartbox/v1/boxes/{box_id}/ack | Ingestion worker | Assigned device | QoS 1, non-retained; committed sample ID. |

Broker access controls restrict every device to its own sample/heartbeat publications and acknowledgement/challenge subscriptions. Use individual client certificates, validate the broker certificate, and revoke compromised identities. The worker acknowledges a sample only after MySQL commits it; broker PUBACK does not prove database storage. Identical retries use the original ID/content; conflicting reuse is rejected. Persist the sequence across restarts. Delayed samples do not update live contact; fresh heartbeats use an expiring single-use server challenge and are not queued offline. These controls are proposals to test before deployment. See Mosquitto configuration [source].

## 4. Data contract, storage and processing

Every 30 seconds, persist an attempt locally before publishing. Heartbeats occur every 60 seconds. Store UTC times and render clearly labelled local times in the interface. At least 2,880 pending attempts cover 24 hours. Reserve storage headroom, monitor queue capacity and show storage failure rather than silently discarding pending data.

{

"schema_version": 1,

"sample_id": "32cb1c3f-097f-41d5-9e99-85d98c3da2b1",

"box_id": "box-001",

"sequence_no": 42,

"profile_id": "539ca70a-17ca-40d0-8884-c03fe65a530b",

"observed_at": "2026-10-09T10:00:00Z",

"clock_reliable": true,

"temperature_c": 24.2,

"quality": "valid"

}

For sensor errors use temperature_c: null and quality: "sensor_error"; for unreliable clocks use a null observation time and a false clock flag. The server adds receipt time and derives the recorded band using the immutable profile. Validate topic identity against the payload box, types, schema version, size, sequence, finite values and profile ownership. Preserve observation and receipt times separately.

The database research supplies relational tables, foreign keys and parameterised queries; schema.sql extracts that proposed schema for review. auth_subject is the login identifier; password_hash stores the Argon2id hash for the proposed local-account design. This is a core schema proposal, not a tested migration. Session storage, device certificate registration, notification subscriptions/outbox and job-run records require additional tables during implementation. Boxes link to users through box_access; each sample links to the profile for that box. Optional review records do not alter measurements. Local SQLite stores pending messages and the persistent next sequence atomically.

Let L < W < U. A valid temperature below L or above U is Alert; W through U is Warning; L through values below W is Normal. Demonstration W = 24°C and U = 25°C; L must be agreed before tests. The LED is off for Normal and steadily lit for Warning/Alert. Proposed sensor-error feedback is two short flashes followed by a pause. Application colour is supplemented by words and symbols. The buzzer remains Should Have.

Proposed stale-reading threshold: 90 seconds; proposed device-contact timeout: 180 seconds. Present the recorded band separately from freshness. An old Normal reading cannot establish current Normal conditions. Latest state follows persistent device sequence, so delayed lower-sequence uploads enrich history without replacing current state. Unreliably timed readings remain explicitly unknown in timed history. Estimated excursions are optional: break intervals at missing/error samples and label duration as an estimate.

### Scheduled processing with cron

Sampling and immediate local warnings run in the device loop, not cron. On a UTC-configured cloud host, a restricted service account runs these proposed jobs:

* * * * * /usr/bin/flock -n /run/smartbox/stale.lock /opt/smartbox/.venv/bin/python /opt/smartbox/jobs/check_stale.py

*/5 * * * * /usr/bin/flock -n /run/smartbox/summary.lock /opt/smartbox/.venv/bin/python /opt/smartbox/jobs/rebuild_summaries.py

15 2 * * * /usr/bin/flock -n /run/smartbox/backup.lock /opt/smartbox/.venv/bin/python /opt/smartbox/jobs/backup.py

45 2 * * * /usr/bin/flock -n /run/smartbox/retention.lock /opt/smartbox/.venv/bin/python /opt/smartbox/jobs/apply_retention.py

| Job | Data processing |
| --- | --- |
| Every minute | Check authenticated live contact age; create one event for a transition to not reporting. |
| Every five minutes | If FR-13 is implemented, recalculate affected estimated summaries after delayed observations; break at gaps/errors. |
| Daily at 02:15 UTC | Create an encrypted backup, record outcome and copy off-host; separately test restoration. |
| Daily at 02:45 UTC | Apply agreed retention in bounded batches only after a verified successful backup. |

Paths and scripts are deployment examples, not implemented jobs. Provision writable lock directories and log errors. Locks prevent overlapping runs; jobs must be idempotent. The retention job must check the recorded backup result rather than assume that running later means the backup succeeded.

Provision lock directories with service ownership. Stale checks create one transition event per outage. Summary jobs recompute affected periods after delayed uploads. Backups are encrypted and copied off-host; test restoration. Proposed prototype retention is 90 days for samples and 14 daily backups, subject to team agreement. Retention uses bounded transactions and runs only after a successful backup. Log job outcomes and surface failures to maintainers.

## 5. Security and privacy

| Threat | Proposed control | Verification |
| --- | --- | --- |
| Device impersonation or cross-box publishing | Individual client certificates, broker topic ACLs and payload/topic identity checks; revocation procedure. | Attempt wrong certificate, wrong topic and forged box ID. |
| Interception | Certificate-validated MQTT/TLS and HTTPS; no plaintext fallback. | Reject untrusted/expired certificates. |
| Unauthorised history access | Server-side box authorisation on every read/export/review operation. | Assigned and unassigned API tests. |
| Account compromise | Argon2id password hashes, rate-limited login, generic failure messages; Secure/HttpOnly/SameSite cookies, session expiry and server-side logout invalidation. | Authentication, expiry and session-reuse tests. |
| Injection and browser attacks | Parameterised SQL, validated request schemas, output escaping, CSRF protection for cookie-authenticated mutations. | Malformed input and CSRF tests. |
| Device theft or exposed credentials | Locked enclosure where feasible, restricted OS accounts, protected key files, updates and immediate credential revocation; keep secrets out of Git. | Inspect permissions and test revocation. Physical access remains a risk. |
| Database or backup disclosure | Private database endpoint, least-privilege service user, host/storage encryption and encrypted backups. | Inspect configuration and restore with authorised credentials. |
| Duplicate/replayed data | Stable sample IDs and content comparison, persistent sequence, explicit application acknowledgement and fresh heartbeat challenge. | Duplicate, replay and delayed-upload tests. |

## 6. UI and notifications

TempSafe is planned as a responsive web application with a Python/FastAPI backend and an HTML, CSS and JavaScript frontend. Planned Progressive Web App (PWA) support would add a home-screen icon and standalone launch on supported phones. PWA installation and Web Push require browser/platform testing. A separate native mobile application is likely to follow the web application, subject to a final team decision.

UI concepts and interaction design show sign-in, current status and history. They use synthetic values and are design artefacts, not a functioning application. The authenticated user sees only their assigned box. Display temperature/unit, recorded status, observation time, last contact and freshness separately. Provide a date range, graph and equivalent table, with visible gaps and error labels. Empty, denied-access, stale, sensor-error and offline states need explicit wording. Use labelled fields, visible focus, keyboard navigation, text/symbol warnings and readable contrast.

Proposed phone channel: browser Web Push with a service worker and explicit permission. Confirm browser/platform support during implementation; a denied or unsupported permission must be visible. The worker commits a notification outbox item with each eligible current-state transition, then retries delivery separately. Suppress unchanged repeated states, prevent duplicate outbox events and mark provider failures. Historical uploads do not trigger current warnings. The proposed target is receipt within 60 seconds of an online Warning/Alert transition; measure it on supported phones rather than claiming guaranteed delivery. Subscribers receive notifications only for authorised boxes. Notification text links to the authenticated dashboard; it does not expose sensitive account information. On iOS/iPadOS, Web Push requires adding the web application to the home screen. This access remains a web application.

## 7. Testing and success criteria

The initial test plan defines boundary, authorisation, sensor, offline, duplicate, history and accessibility cases. Add automated API tests for certificate/topic rejection, schema validation and cross-box requests; browser tests for sign-in, history filters, keyboard navigation and stale-state rendering.

Invite intended users where available; otherwise identify participants as proxies rather than paramedics. Ask participants to sign in, identify a warning, distinguish stale data and find an earlier flagged sample without coaching. Record task completion, time, errors and feedback. Proposed success targets: every participant distinguishes stale data from current Normal; at least 80% complete the core journey unaided; all Must Have functional cases pass; 24-hour queue reconciliation shows no unexplained loss/duplicates.

## 8. Responsibilities

| Member | Proposed G-CA1 work | Implementation responsibility |
| --- | --- | --- |
| Nikita Smiichyk | Review architecture, data schema, processing, security and test plan. | Backend, database and automated tests. |
| Hanna Bokariuk | Verify hardware inventory/wiring, procurement, persona and Universal Design decisions. | Hardware, calibration and user evaluation. |
| Maryna Hordiienko | Review UI wireframes, interaction flows and accessibility acceptance criteria. | Frontend, Web Push integration and browser tests. |

## 9. Assessment traceability

| Criterion | Evidence |
| --- | --- |
| Documentation (20%) | This design, linked scope/requirements and individually identifiable review commits. |
| Hardware (20%) | Parts list, wiring SVG/PNG, manufacturer references, power/procurement discussion. A native Fritzing diagram. |
| Data/storage/processing (20%) | Data contract, database schema/queries, queue semantics and explicit cron entries. |
| Security/privacy (10%) | Threat/control/test table and secure pub-sub design. |
| UI/users/testing (20%) | Wireframes, persona, user stories and functional/automated/end-user test proposals. |
| Version control (10%) | Accessible repository and genuine incremental contributions. |

## 10. References

Access date for web references: 9 October 2026.

Sensirion (2023), SCD4x datasheet, version 1.5[source], sections 3.5–3.6: periodic readings and temperature compensation.

Adafruit (n.d.), SCD-4x pinouts[source]: breakout connections.

Raspberry Pi (n.d.), Raspberry Pi 400 specifications[source]: controller, wireless connectivity and power.

Eclipse Mosquitto (n.d.), mosquitto.conf manual[source]: proposed broker certificate and access controls.

SQLite (n.d.), Appropriate uses[source]: embedded local storage rationale.

MySQL (n.d.), InnoDB documentation[source]: central relational storage rationale.

Beams, C. (n.d.), How to Write a Git Commit Message[source]: commit-writing guidance.

HPRA / Mercury Pharmaceuticals (2024), Fentanyl injection leaflet[source], section 5: one product-specific storage example. It does not establish a universal medicine range.

# Appendix A Hardware and wiring

# IoT Controlled Drug Box – Development Parts List

This parts list reflects the current TempSafe project scope: continuous temperature monitoring inside a prototype controlled drug box using a Raspberry Pi and SCD-41 sensor, with a single-colour warning LED. A buzzer is a Should Have enhancement, subject to component availability and testing.

## Parts List

| Part | Purpose / Notes |
| --- | --- |
| Raspberry Pi 400 | Main controller. Collects temperature readings, processes the temperature status, stores data and sends it to the web application. It controls the LED indicator and, if implemented, the buzzer. |
| Adafruit SCD-41 | Already owned. I²C sensor with integrated temperature and humidity measurement; only temperature is used. Humidity and CO₂ reporting are outside this release. |
| Single-colour LED | Provides immediate visual feedback without requiring the Advanced Paramedic to open the web application. Lights for Warning or Alert; off for Normal; distinct blinking for sensor failure. No RGB capability is required. |
| Active buzzer | Should Have: provides an audible alert on Alert, with a silence control; not a guaranteed baseline component. |
| LED resistor (proposed 1 kΩ) | Protect the LED(s) by limiting current. The wiring drawing proposes 1 kΩ; verify brightness and current against the actual LED specifications. |
| Full-size breadboard | Used to build and test the circuit without soldering. |
| Jumper wires | Connect the SCD-41, LED indicator, buzzer and breadboard to the Raspberry Pi. Male/Male and Female/Male wires are useful for prototyping. |
| microSD card | Stores Raspberry Pi OS, project software and local log data. |
| Raspberry Pi 400 power supply | Mains power for the bench prototype; the Pi communicates over Wi-Fi and does not need a direct connection to the laptop. |
| Plastic prototype box | One simple plastic storage box for temperature monitoring and local feedback. No real controlled drugs will be used. |

## Feedback Logic

Normal: physical LED off; application shows green with a text label and symbol.

Warning: physical LED on; application shows amber with text and a symbol; send a phone notification when connected.

Alert: physical LED on; application shows red with text and a symbol; send a phone notification when connected. Sound the buzzer if implemented.

Sensor failure: distinct LED blinking and an explicit application error, never Normal.

For the existing demo example, Warning starts at 24°C and high-temperature Alert starts above 25°C. Configured lower-limit violations also produce Alert.

The LED and buzzer are output / feedback devices, not sensors. The SCD-41 remains the main sensor used to collect environmental data.

## Scope Notes

Scope note: The reed switch/opening sensor and GNSS/GPS module are not included in the current version of the project.

Project focus: The prototype focuses on temperature monitoring over time, logging, web alerts and phone notifications, and clear local feedback through light and sound.

Temperature threshold: The prototype uses 25°C as the upper temperature limit for alerting, with an early warning before the limit is exceeded.

User interaction: The Advanced Paramedic should not need to check the application while the temperature is normal. The system provides feedback automatically when attention is required.

# Prototype wiring — Raspberry Pi 400 and Adafruit SCD-41

## Sensor connections

| Raspberry Pi 400 physical pin | Function | Adafruit SCD-41 breakout connection |
| --- | --- | --- |
| 1 | 3.3 V power | VIN |
| 6 | Ground | GND |
| 3 | GPIO2 / SDA1 | SDA |
| 5 | GPIO3 / SCL1 | SCL |

## Proposed LED connections

Physical pin 11 (BCM GPIO17) → 1 kΩ series resistor → LED anode; LED cathode → physical pin 9 (GND). This matches the SVG/PNG drawing. Verify the actual LED ratings, polarity and brightness before powering the circuit. The resistor value is a proposal, not a measured result.

## What each part does

| Part | Role |
| --- | --- |
| Plastic prototype box | Holds the sensor; the container itself has no electrical connection. |
| Adafruit SCD-41 breakout | Measures temperature inside the box; humidity and CO₂ reporting are outside this release. |
| Raspberry Pi 400 | Reads the sensor, runs the monitoring software and controls the warning LED. |
| Solderless breadboard | Provides temporary electrical connections without soldering; it has no processor or Wi-Fi. |
| Jumper wires | Carry power, ground and signals between components. Colours in the diagram are labels, not a required colour standard. |
| Resistor | Limits current through the warning LED. |
| Single-colour LED | Provides local visual feedback. It is an output device, not a sensor. |
| USB-C power supply | Powers the Pi 400 from mains; use a compatible 5 V / 3 A supply. |
| Laptop | Provides remote access through Raspberry Pi Connect; no direct laptop-to-Pi cable is required. |

# Appendix B Database schema and queries

# Smart Box: Database Technology Research

Scope: one internal temperature sensor, a single-colour warning LED and Must Have phone notifications for a paramedic medication box. The buzzer is Should Have; humidity reporting is outside this release.

The device records temperature inside Controlled Drug Box. MySQL is the proposed central database; SQLite provides a durable offline queue on the Raspberry Pi.

The baseline uses one internal sensor. An external temperature sensor is a possible future research extension and is not included in this schema.

## 1. System technologies and languages

| Component | Proposed technology | Language or format | Role in Smart Box |
| --- | --- | --- | --- |
| Device application | Raspberry Pi application | Python | Read one internal temperature sensor; timestamp and queue samples |
| Device's local storage | SQLite | SQL through Python | Keep an offline upload queue on the device |
| Network messages | MQTT over TLS | JSON payloads | Publish observations through the broker to the ingestion worker |
| Backend | FastAPI | Python | Authenticate devices and users, validate messages and retrieve data |
| Database connector | MySQL Connector/Python | Python | Let the backend execute parameterised MySQL queries |
| Central database | MySQL with InnoDB | SQL | Store boxes, access permissions and historical observations |
| User interface | Web application | HTML, CSS and JavaScript | Display temperature, freshness, history and warnings |
| Scheduled processing | cron running application scripts | cron syntax and Python | Produce summaries, check stale devices and perform maintenance |

## 2. Data requirements

| Data | Example fields | Proposed policy | Purpose |
| --- | --- | --- | --- |
| Box metadata | Box ID, name, last contact | Provisioned once; contact updated on authenticated device requests | Identify the monitored box and connection freshness |
| Temperature samples | Sample ID, sequence number, temperature Celsius, quality | Every 30 seconds, including explicit failed sample attempts | Current state and history |
| Timing | Observed-at UTC, received-at UTC, clock reliability | Every sample; preserve original values on retry | Distinguish historical uploads and uncertain timing |
| Threshold profiles | Profile ID, lower/upper limits, warning threshold, source note, demo flag | Immutable version for each configuration change | Reproduce the interpretation applied when recorded |
| User access | User ID and permitted box IDs | On provisioning/change | Enforce authorisation |
| Review acknowledgement | Sample ID, reviewer, reviewed-at | Optional Should Have workflow | Record review without altering measurements |

Heartbeats are sent every 60 seconds. Proposed contact timeout is 3 minutes. The device needs at least 24 hours of local queue capacity (2,880 sample attempts).

## 3. Database categories

| Category | Examples | Data model | Smart Box relevance |
| --- | --- | --- | --- |
| Relational | MySQL, PostgreSQL, SQLite | Tables linked by keys | Natural fit for boxes, users, permissions, threshold profiles and timestamped samples |
| Document | MongoDB, Cloud Firestore | Collections of structured documents | Useful when sensor payloads vary significantly |
| JSON tree | Firebase Realtime Database | Hierarchical JSON | Convenient for synchronising a small current-state view |
| Time-series | InfluxDB | Timestamped measurements | Useful if high-volume sensor analytics becomes the main requirement |
| Key-value/data structures | Redis | Values and specialised structures accessed by keys | Possible cache or transient state store |
| Graph | Neo4j | Nodes and relationships | Useful for relationship traversal; not a core need here |

## 4. Main comparison: MySQL, MongoDB and Firebase

| Criterion | MySQL / InnoDB | MongoDB | Cloud Firestore | Firebase Realtime Database |
| --- | --- | --- | --- | --- |
| Model | Relational tables | BSON documents | Collections and documents | JSON tree |
| Example design | boxes, temperature_samples, box_access | Box and temperature-sample collections | Box documents with sample subcollections | Separate box-state and history paths |
| Relationships | Foreign keys and SQL joins | References or embedding; application controls references | Document references; access patterns often require duplicated fields | Tree paths and duplicated data for different access patterns |
| Integrity | Transactions and foreign-key constraints | Validation and transactions; no SQL-style foreign-key enforcement | Transactions and validation through backend/rules | Transactions and validation rules |
| Historical reporting | SQL filters, joins and aggregation | Query and aggregation pipelines | Indexed queries; report design may need extra processing | Limited query model; historical reports need more application work |
| Live dashboard | Backend polling, SSE or WebSockets | Backend updates; change streams where supported | SDK listeners | SDK listeners and native presence features |
| Offline device collection | Build a local queue | Build a local queue | Do not assume web/mobile SDK caching covers a Python device process | Do not assume client synchronisation replaces durable Python device buffering |
| Operations | Operate a server or use a managed host | Self-host or use a managed deployment | Managed cloud service | Managed cloud service |
| Main trade-off here | Schema and server maintenance | More application responsibility for relationships | Provider-specific modelling, query and billing behaviour | Awkward growing history and cross-entity reporting |
| Decision | Proposed central database | Viable alternative | Viable alternative for a Firebase-focused team | Possible current-state solution; not preferred for primary history |

## 5. Other options and why they are not selected

| Technology | Could it work? | Decision and reason |
| --- | --- | --- |
| PostgreSQL | Yes; a strong relational alternative | Suitable for this same schema and reporting workload. MySQL is retained for consistency with the existing project proposal; spatial extensions are unnecessary. |
| SQLite | Yes, including small backend deployments | Select it for the local device queue. It needs no separate database server. A shared central deployment with many concurrent writers is less attractive because writes are serialised per file; this does not mean SQLite is incapable of running a small web application. |
| InfluxDB | Yes, for time-series telemetry | Reconsider for dense measurement streams and time-based analytics. It adds another modelling/operational choice without a clear benefit for the small temperature-sampling workload. |
| Redis | Technically possible, with an appropriate design | Do not use it as the sole historical record here. It supports persistence, so “Redis cannot save data” is false. Persistence settings have durability trade-offs; a cache would be an optional later role. |
| Neo4j | Yes, but poorly matched | The prototype asks for state and history, not multi-hop relationship traversal. A graph model adds learning and modelling work without solving a current requirement. |
| CSV or JSON files | Suitable for exports and fixtures | Not chosen as the shared authoritative store: locking, atomic changes, integrity checks, access management and efficient searching would need substantial custom code. |

## 6. Proposed decision

Recommendation: MySQL as the central database, SQLite as the device's durable upload queue, and a Python/FastAPI backend.

MySQL fits structured relationships between boxes, users, access permissions and threshold profiles, as well as time-filtered reporting. The proposed rate of one sample every 30 seconds produces 2,880 samples per box per day. This workload does not by itself require a specialised time-series database. PostgreSQL would also be a sound choice; MySQL is retained for continuity with the team's existing proposal, not because alternatives cannot perform the task.

SQLite has a separate responsibility: preserve pending samples on the device when the network is unavailable. It is not a second competing central database. Proposed intervals are starting values for testing, not medical requirements.

## 7. Proposed relational schema and queries

This core schema is a proposal, not an executed migration. auth_subject stores the local login identifier and password_hash its Argon2id hash. Session, device-certificate, notification subscription/outbox and maintenance tables remain implementation work. G-CA1 design defines proposed cron jobs, 90-second freshness, Web Push and retention settings.

CREATE TABLE boxes

(

box_id       VARCHAR(64) PRIMARY KEY,

name         VARCHAR(100) NOT NULL,

last_seen_at DATETIME(6) NULL

) ENGINE=InnoDB;

CREATE TABLE app_users

(

user_id      BIGINT UNSIGNED AUTO_INCREMENT PRIMARY KEY,

auth_subject VARCHAR(191) NOT NULL UNIQUE,

password_hash VARCHAR(255) NOT NULL

) ENGINE=InnoDB;

CREATE TABLE box_access

(

user_id BIGINT UNSIGNED NOT NULL,

box_id  VARCHAR(64) NOT NULL,

PRIMARY KEY (user_id, box_id),

FOREIGN KEY (user_id) REFERENCES app_users (user_id),

FOREIGN KEY (box_id) REFERENCES boxes (box_id)

) ENGINE=InnoDB;

CREATE TABLE threshold_profiles

(

profile_id  CHAR(36) PRIMARY KEY,

box_id      VARCHAR(64)   NOT NULL,

lower_c     DECIMAL(5, 2) NOT NULL,

upper_c     DECIMAL(5, 2) NOT NULL,

warning_c   DECIMAL(5, 2) NOT NULL,

source_note TEXT          NOT NULL,

is_demo     BOOLEAN       NOT NULL DEFAULT TRUE,

created_at  DATETIME(6) NOT NULL DEFAULT CURRENT_TIMESTAMP(6),

UNIQUE KEY uq_profile_box (profile_id, box_id),

FOREIGN KEY (box_id) REFERENCES boxes (box_id),

CHECK (lower_c < warning_c AND warning_c < upper_c),

CHECK (is_demo IN (0, 1))

) ENGINE=InnoDB;

CREATE TABLE temperature_samples

(

sample_id      CHAR(36) PRIMARY KEY,

box_id         VARCHAR(64) NOT NULL,

sequence_no    BIGINT UNSIGNED NOT NULL,

profile_id     CHAR(36)    NOT NULL,

observed_at    DATETIME(6) NULL,

clock_reliable BOOLEAN     NOT NULL,

received_at    DATETIME(6) NOT NULL DEFAULT CURRENT_TIMESTAMP(6),

temperature_c  DECIMAL(5, 2) NULL,

quality        ENUM('valid','sensor_error') NOT NULL,

UNIQUE KEY uq_box_sequence (box_id, sequence_no),

KEY            ix_box_time (box_id, observed_at),

FOREIGN KEY (box_id) REFERENCES boxes (box_id),

FOREIGN KEY (profile_id, box_id)

REFERENCES threshold_profiles (profile_id, box_id),

CHECK (clock_reliable IN (0, 1)),

CHECK ((clock_reliable = 1 AND observed_at IS NOT NULL)

OR (clock_reliable = 0 AND observed_at IS NULL)),

CHECK ((quality = 'valid' AND temperature_c IS NOT NULL

AND temperature_c BETWEEN -40 AND 80)

OR (quality = 'sensor_error' AND temperature_c IS NULL))

) ENGINE=InnoDB;

-- Should Have

sample_id   CHAR(36) PRIMARY KEY,

reviewed_by BIGINT UNSIGNED NOT NULL,

reviewed_at DATETIME(6) NOT NULL DEFAULT CURRENT_TIMESTAMP(6),

FOREIGN KEY (sample_id) REFERENCES temperature_samples (sample_id),

FOREIGN KEY (reviewed_by) REFERENCES app_users (user_id)

) ENGINE=InnoDB;

### Latest sample for an authorised user

SELECT s.sample_id,

s.sequence_no,

s.temperature_c,

s.quality,

s.observed_at,

s.clock_reliable,

s.received_at,

p.lower_c,

p.upper_c,

p.is_demo,

b.last_seen_at,

CASE

WHEN s.quality <> 'valid' THEN 'unknown'

WHEN s.temperature_c < p.lower_c OR s.temperature_c > p.upper_c THEN 'Alert'

WHEN s.temperature_c >= p.warning_c THEN 'Warning'

ELSE 'Normal' END AS recorded_band

FROM temperature_samples s

JOIN threshold_profiles p ON p.profile_id = s.profile_id

JOIN boxes b ON b.box_id = s.box_id

JOIN box_access a ON a.box_id = s.box_id

WHERE a.user_id = %s AND s.box_id = %s

ORDER BY s.sequence_no DESC

LIMIT 1;

### Temperature history

SELECT s.sample_id,

s.observed_at,

s.temperature_c,

s.quality,

s.sequence_no,

p.lower_c,

p.upper_c

FROM temperature_samples s

JOIN threshold_profiles p ON p.profile_id = s.profile_id

JOIN box_access a ON a.box_id = s.box_id

WHERE a.user_id = %s

AND s.box_id = %s

AND s.clock_reliable = 1

AND s.observed_at >= %s

AND s.observed_at

< %s

ORDER BY s.observed_at, s.sequence_no;

### Out-of-range observations

SELECT s.sample_id,

s.observed_at,

s.temperature_c,

p.lower_c,

p.upper_c,

r.reviewed_at

FROM temperature_samples s

JOIN threshold_profiles p ON p.profile_id = s.profile_id

JOIN box_access a ON a.box_id = s.box_id

LEFT JOIN sample_reviews r ON r.sample_id = s.sample_id

WHERE a.user_id = %s

AND s.box_id = %s

AND s.quality = 'valid'

AND (s.temperature_c

< p.lower_c

OR s.temperature_c

> p.upper_c)

ORDER BY s.sequence_no DESC;

### Local SQLite queue

Persist the sample ID, box ID, sequence, profile version, observation time/clock flag, value/quality and delivery status in one local transaction before uploading. Store the next sequence safely. Remove or mark delivered only after server acknowledgement. A test at 30-second sampling must reconcile at least 2,880 sample attempts over 24 hours and survive a normal restart. Report storage errors/full queues; no silent overwriting of pending data. Profile configuration is retained locally so offline warnings still work.

## 8. Processing, security and privacy

### Processing placement

Device loop: sample every 30 seconds, validate, classify and control local LED without internet; heartbeat every 60 seconds when connected.

MQTT ingestion worker: enforce authenticated topic/box identity, validate and deduplicate, preserve historical times and profiles; acknowledge only after database commit. HTTPS serves user-facing API requests.

Dashboard: show current freshness, raw history, threshold bands and flagged observations; do not rely on cron for the local warning.

### Security

Per-device credentials, revocation, MQTT over TLS and HTTPS, message/request validation and rate limiting; no central DB password on a Pi or in a browser.

Application authentication plus box-level authorisation on every read, review and history endpoint; protected sessions and password handling or a managed identity provider.

Restricted DB network access, protected connections, least-privilege database accounts, backups and tested restore procedure.

Keep secrets out of Git; protect physical wiring, Pi storage and configuration against casual access.

Collect no patient details, staff movement history or medication administration records. User IDs are retained only for access and optional review attribution.

Ordinary database records are not a tamper-proof clinical audit trail. This is an educational prototype, not a certified medicine-quality assessment system.

## 9. Learning resources and references

| Ref | Resource | What to use it for |
| --- | --- | --- |
| R1 | FastAPI: SQL databases [source] | Understand backend/database integration |
| R2 | MySQL Connector/Python Developer Guide [source] | Python connections, parameter binding and transactions |
| R3 | MySQL: Introduction to InnoDB [source] | Transactional storage and relational integrity |
| R4 | MySQL: Optimisation and indexes [source] | Choose indexes from query patterns |
| R5 | MongoDB: Schema validation [source] | Validate flexible documents |
| R6 | MongoDB: Transactions [source] | Understand transaction capabilities and constraints |
| R7 | Firebase: Choose a database [source] | Distinguish Firestore from Realtime Database |
| R8 | Firebase pricing [source] and Firestore billing [source] | Estimate cost using the chosen service and workload |
| R9 | SQLite: Appropriate uses [source] | Understand embedded storage and concurrency trade-offs |
| R10 | InfluxDB 3 Core documentation [source] | Evaluate a time-series-focused alternative |
| R11 | Redis persistence [source] | Understand durability choices before using Redis for stored state |
| R12 | Neo4j: Graph database concepts [source] | Understand when graph modelling is useful |

# Appendix C UI mock-ups and interactions

# Smart Box: user interface design

Status: design proposal for G-CA1; synthetic data, not a running application.

## Sign-in and access

TempSafe is planned as a responsive web application with a Python/FastAPI backend and an HTML, CSS and JavaScript frontend. Planned Progressive Web App (PWA) support would add a home-screen icon and standalone launch on supported phones. PWA installation and Web Push require browser/platform testing. A separate native mobile application is likely to follow the web application, subject to a final team decision.

Users open the HTTPS web application on a phone or desktop browser. Sign-in has labelled login-identifier and password fields, a Sign in button and a generic failed-login message. Successful sign-in opens the user's assigned prototype box; an unassigned account sees an explicit no-access message. Sign out invalidates the server session. All box/history requests are authorised by the backend; hiding a button is not access control.

## Current status concept

Figure 1. Existing team concept for the box dashboard. The excursion timer and maximum summary are optional Should Have features. “Above safe limit” is concept wording: the implementation must say “Above configured upper limit”, without implying a clinical assessment. The actual prototype has one box; vehicle labels and medicines in the image are illustrative only.

The baseline screen must add separate observation and last-contact times, profile limits/version and freshness. Display a temperature in °C and recorded Normal/Warning/Alert with text and symbols. If older than the proposed 90-second threshold, show Data stale and state that current conditions are unknown. A failed latest attempt shows Sensor error; a previously valid value may remain visible only as historical information. Never show an old Normal value as current proof of suitable conditions.

The View temperature history button opens the history screen. Notifications are optional for the user to enable through Web Push permission; the monitoring feature remains accessible if permission is denied, while the UI explains that phone alerts are unavailable.

## History concept

Figure 2. Existing synthetic history concept. The time/maximum summary is conditional on FR-13. The domain shown is a mock-up label, not a deployed service. The continuous graph illustrates complete synthetic observations; missing data in the implementation must appear as gaps.

The baseline history screen includes a start/end date-time filter and an equivalent readable table with observation time, temperature, quality, recorded band and profile limits. Invalid intervals have a clear error; an empty range says No observations found. Original observation times are used even after delayed upload; clock-unreliable attempts are listed separately with an explicit timing warning. A sample-review control is conditional on FR-12 and does not clear an ongoing warning.

## Required states and interactions

| State | Proposed wording/behaviour |
| --- | --- |
| No measurements | No observations yet; current conditions unknown. |
| Valid current data | Value/unit, recorded status, profile, observation time and last contact. |
| Stale reading | Data stale; last value is historical; current conditions unknown. |
| Sensor failure | Sensor error; do not replace the failed sample with zero or Normal. |
| No live contact for 180 seconds | Device not reporting; show last authenticated live contact separately from reading age. |
| Network/API failure | Unable to refresh; retain visibly dated information and offer retry. |
| No box permission | No assigned box available; do not expose another user's data. |
| Web Push unsupported/denied | Phone notifications unavailable; explain permission/support state. |

Controls have visible focus, keyboard access, meaningful labels and a logical tab order. Colour is supplemented by words and symbols. Provide a table alternative to the graph, support narrow screens and 200% zoom, and check contrast. The test plan defines the planned accessibility and user tasks; no application execution results are claimed.

# Appendix D Persona

# Persona – TempSafe

Name: Shannon Rice

Age: 32

Occupation: Advanced Paramedic

Experience: 7 years in pre-hospital emergency care

Location: Ireland

User type: Confident smartphone and workplace digital-system user, but not a technical expert.

## About

Shannon works as an Advanced Paramedic and regularly responds to emergency calls.

During her shift, controlled drugs are stored in a secure Controlled Drug Box inside a Vehicle Security Safe in the ambulance.

Shannon often works in stressful and time-critical situations. While she is treating a patient, she may not be able to return to the parked ambulance to check the drug box or change the vehicle conditions.

TempSafe is designed to monitor the temperature inside the Controlled Drug Box automatically over time and provide clear feedback only when attention is required.

## Goals

Shannon wants to:

make sure temperature-sensitive controlled drugs remain within suitable storage conditions;

avoid spending time on regular manual temperature checks;

receive an early warning if the temperature is approaching the configured limit;

know how long the medication has been exposed to an elevated temperature;

quickly understand whether a temperature excursion has occurred;

receive clear feedback without needing to open the web application on her phone every time;

receive only important notifications that do not distract her from emergency care.

## User Needs

The Advanced Paramedic needs a system that:

works automatically in the background;

continuously monitors temperature over time;

does not require regular user interaction while conditions are normal;

records timestamped out-of-range observations; estimated duration and maximum-temperature summaries are Should Have;

provides clear visual feedback directly on the box;

can add an audible warning if the Should Have buzzer is implemented;

sends a phone notification when the temperature approaches or exceeds the configured limit;

uses clear text as well as colour so that the status is not communicated by colour alone;

is simple and reliable to use under pressure.

## Frustrations

Shannon may experience the following problems:

she cannot leave a patient during emergency care simply to check the temperature of a box in the ambulance;

the temperature inside a parked ambulance can rise significantly even when the outside temperature is moderate;

the outside air temperature does not show the actual conditions inside a closed vehicle safe;

without continuous monitoring, she may not know how long the medicines were exposed to a high temperature;

regular manual checks take additional time;

too many unnecessary notifications may be distracting;

a status shown only by colour may be unclear or inaccessible;

complicated interfaces are difficult to use in emergency situations.

## Technology Use

Shannon regularly uses a smartphone and digital systems at work, but TempSafe should require minimal interaction.

If the storage conditions remain normal, Shannon should not need to check or enter anything manually. The system should monitor the box continuously and provide local feedback through an LED indicator.

The web application is mainly used when Shannon receives a warning or alert, or when she needs to review the temperature history and duration of an excursion.

## Typical Scenario

During an emergency call, Shannon is treating a patient away from the ambulance. The ambulance is parked outside in direct sunlight. The outside temperature may be moderate, for example around 18°C, but the temperature inside the vehicle and the enclosed storage area can rise much higher.

Shannon cannot leave the patient to return to the ambulance and manually check the Controlled Drug Box or turn on the air conditioning.

TempSafe continuously monitors the temperature inside the box and records how the temperature changes over time.

While the temperature remains normal, the system requires no action. The single-colour LED is off; the application shows Normal with text and a symbol.

With connectivity and notification permission, if the temperature reaches the early-warning range, the single-colour LED lights and TempSafe sends a phone notification with a clear message, for example:

> Warning: Controlled Drug Box temperature has reached 24°C.

If the temperature exceeds 25°C, the LED remains lit and, if implemented, the buzzer provides an audible alert, and the system records the temperature excursion. With connectivity and permission, a phone Alert is also sent:

> Alert: Controlled Drug Box temperature has exceeded 25°C. > If the Should Have excursion summary is implemented, the web application can show an estimated period and maximum temperature, for example:

> ESTIMATED EXCURSION – 12 min – max 27.4°C

The baseline records individual observations for later review. Estimated summaries must break at gaps and invalid samples; they do not prove continuous exposure.

## Environment

TempSafe is designed specifically for use in an ambulance.

The Controlled Drug Box is kept inside a Vehicle Security Safe in the vehicle. Because this is an enclosed storage area, its temperature can differ significantly from the outside air temperature.

Research on medication storage in emergency vehicles has shown that medication storage areas can become considerably hotter than the outdoor environment. In one study, the temperature inside an insulated drug pouch reached more than 40°C.

For this reason, TempSafe measures the temperature directly inside the Controlled Drug Box rather than relying on the outside temperature.

## Tasks

The Advanced Paramedic should not need to perform regular manual temperature checks. TempSafe should automatically:

measure the temperature inside the Controlled Drug Box;

record temperature readings over time;

operate without user input while conditions are normal;

provide a single-colour local LED warning and application status text with symbols;

send an early phone warning when the temperature approaches the configured limit;

send an Alert when connected and, if implemented, use the optional buzzer locally;

retain out-of-range observations; offer estimated timing/maximum summaries if implemented;

allow the user to review the recorded event later.

## Main Use Case

TempSafe continuously monitors temperature over time inside the Controlled Drug Box located in the ambulance security safe.

While conditions are normal, the system works in the background and requires no action from Shannon.

If the temperature approaches the configured limit, TempSafe provides an early warning through the LED indicator and a phone notification. If the temperature exceeds the limit, it displays Alert and retains the observations; optional buzzer and estimated summaries can add feedback.

The aim is to give the Advanced Paramedic useful information at the right time without adding unnecessary manual checks during patient care.

## System

The TempSafe system includes:

Controlled Drug Box – secure container used to store controlled drugs;

Vehicle Security Safe – secure safe inside the ambulance where the Controlled Drug Box is kept;

Raspberry Pi – collects and processes sensor data;

SCD-41 temperature sensor – measures the temperature inside the Controlled Drug Box;

LED indicator – provides immediate visual feedback about the temperature status;

Buzzer – Should Have audible feedback, subject to implementation;

Database / Data Storage – stores sample attempts and profiles; estimated excursion summaries are Should Have;

Web Application – displays current status, freshness, history and alerts; estimated excursion duration is optional;

Notification System – sends warnings and alerts to the user;

Internet Connection – allows data to be transferred from the Raspberry Pi to the application.

## Temperature and Feedback Logic

For valid readings within the agreed lower limit, the demonstration uses the following upper-temperature feedback logic. Values below the configured lower limit also produce Alert; sensor failure is an error state with distinct LED blinking:

Below 24°C – Normal: physical LED off; application Normal;

24°C to 25°C – Warning: physical LED on and early phone warning when connected;

Above 25°C – Temperature excursion: physical LED on, optional buzzer and phone alert when connected.

The 24°C threshold is a project-defined early warning. The 25°C threshold is a demonstration upper limit drawn from the project research; it is not a universal medicine storage range.

Colour is not used as the only form of communication. The web interface also displays a clear text status and the measured temperature, and an optional buzzer provides an additional form of feedback for urgent events.

## Main Project Value

The main purpose of TempSafe is not to give the Advanced Paramedic more data to check. It is designed to reduce manual checking and provide useful feedback only when the storage conditions require attention.

The key information is not only the current temperature, but also temperature over time: whether an excursion occurred, how high the temperature became, and how long it lasted.

TempSafe supports the existing work of the Advanced Paramedic instead of requiring the user to constantly adapt to the technology.

TempSafe is an automatic IoT monitoring system for a Controlled Drug Box that tracks temperature over time and gives clear visual, audible and phone-notification feedback when storage conditions require attention.

# Appendix E Functional requirements

# Smart Box — Functional Requirements and Use Cases

## 1. Users and context (UTSE)

| Element | Proposed definition |
| --- | --- |
| Users | Paramedic / staff member authorised to monitor the single shared prototype box; exact operational responsibility to be validated |
| Tasks | Check temperature and data freshness, inspect out-of-range observations, review history and acknowledge review |
| Systems | One temperature sensor, Raspberry Pi, local SQLite queue, authenticated MQTT/TLS broker, HTTPS web API, MySQL database and web application |
| Environments | Controlled Drug Box located inside a Vehicle Security Safe in an ambulance; the initial prototype is demonstrated on a bench with no medicines and uses Wi-Fi connectivity. |

## 2. Functional requirements

| ID | Requirement | Priority | Acceptance criteria |
| --- | --- | --- | --- |
| FR-01 | Sign in and sign out | Must | Valid credentials grant a session; invalid credentials do not; sign-out invalidates the session |
| FR-02 | Restrict access to the assigned prototype box | Must | Multiple authorised users can access the same prototype box; UI and API deny access to unassigned users; unknown box IDs expose no data |
| FR-03 | Measure and record internal temperature | Must | Each sample attempt has a unique ID, box ID, persistent sequence number, UTC observation time (or explicit unreliable-clock status), profile ID and quality status; valid readings are recorded in degrees Celsius |
| FR-04 | Show the latest temperature and its freshness | Must | Show value, unit, observation time and last device contact separately; show unknown before any reading and stale when readings are old; a newer sensor error is visible even if the last valid value remains in history |
| FR-05 | Classify valid samples against a configured profile | Must | With limits L < W < U: T < L or T > U is Alert; W <= T <= U is Warning; L <= T < W is Normal. W is the configured early-warning threshold; invalid samples are unknown, never Normal; preserve the applied profile |
| FR-06 | Provide a local LED warning | Must | A single-colour LED lights by the next completed Warning or Alert sample, including without Wi-Fi; Normal turns it off; an invalid read uses a distinct blink pattern; off is not proof of acceptable conditions or working power |
| FR-07 | Display temperature history and out-of-range records | Must | Filter by box and time; provide graph plus readable table; show thresholds, gaps and quality issues; use observation times, not upload times |
| FR-08 | Identify loss of device contact | Must | After the contact timeout show not reporting and the last contact time; never-connected devices have a separate state; contact alone does not make buffered readings fresh |
| FR-09 | Buffer data while offline | Must | At least 24 hours at the configured sampling interval survive a normal device restart and upload after reconnection; original IDs and times are retained; unacknowledged records are not discarded |
| FR-10 | Handle duplicate and delayed uploads | Must | Identical retries create one sample; conflicting reuse of an ID is rejected; old data does not replace the latest state or trigger a misleading current warning |
| FR-11 | Refresh the dashboard automatically | Should | With connectivity, accepted data appears within the proposed five-second polling interval; refresh failure is visible |
| FR-12 | Record that a user has reviewed a flagged sample | Should | Store reviewer and server timestamp for a flagged sample; acknowledgement does not change the measurement or remove an ongoing warning |
| FR-13 | Estimate observed excursion periods | Must | Group consecutive out-of-range observations using observation time and provide an estimated excursion duration. Missing or invalid samples break continuity; delayed data causes affected history to be recalculated. |
| FR-14 | Send phone notifications | Must | With connectivity and notification permission, send a clear Warning or Alert notification to authorised recipients; define and test the phone delivery channel, suppress repeated notifications for an unchanged state, distinguish delayed historical events from current alerts and expose delivery failures |
| FR-15 | Provide an audible buzzer alert | Should | If implemented, sound on Alert, operate locally without Wi-Fi and provide a silence control; silencing does not clear the LED or dashboard state. Define and test the sound pattern and repeat behaviour. |

## 3. Proposed prototype settings

For the existing demo profile, the early-warning threshold is 24°C and the upper limit is 25°C; the lower limit remains to be selected. Boundary tests cover L, W and U, including exact equality. These settings do not define a universal medicine storage range.

| Setting | Proposal |
| --- | --- |
| Sampling interval | 30 seconds |
| Heartbeat interval | 60 seconds; transmitted independently of whether the sensor read succeeds |
| Reading freshness | Proposed 90 seconds; independent of device contact |
| Phone channel | Proposed Web Push; test permission, browser/platform support, required home-screen installation and a 60-second online receipt target |
| Contact timeout | 3 minutes |
| Local buffer | At least 24 hours (2,880 sample attempts at 30-second intervals) |
| Threshold profile | Immutable versioned lower/upper limits, early-warning threshold and source note; configured by the team, not hard-coded as a universal medicine range |

## 4. Use cases

### UC-01 — Sign in and open the prototype box

Actor: Authorised user. Requirements: FR-01, FR-02.

1. User signs in.

2. An assigned user opens the single prototype box dashboard; other assigned users can access the same box.

3. Backend checks permissions on every request.

Invalid credentials, an unassigned user and denied access have explicit UI states.

### UC-02 — Record a sample and provide a local warning

Actor: Device. Requirements: FR-03, FR-05, FR-06, FR-09, FR-10.

1. Read the internal sensor every 30 seconds.

2. Record a valid temperature or a sensor-error status; never substitute zero for failure.

3. Compare a valid value with the provisioned profile and update the local LED.

4. Persist the attempt in SQLite before publishing via MQTT over TLS when connected.

5. The broker authenticates the device; the ingestion worker validates its box/profile, stores the sample idempotently and publishes an application acknowledgement after commit.

A read failure or unsynchronised clock is identified explicitly. A network failure does not stop local sampling or the LED. Loss of electrical power stops the prototype; it is not a fail-safe medical alarm.

### UC-03 — Check current condition

Actor: Authorised user. Requirements: FR-02, FR-04, FR-05, FR-08, FR-11.

1. Open dashboard and check temperature, profile limits and timestamps.

2. View Normal (green), Warning (amber) or Alert (red), with text and symbols as well as colour; show sensor-error and stale states separately.

3. Distinguish a currently received warning from a historical event uploaded late.

No measurements means unknown. A connected device with failed reads is not shown as healthy temperature monitoring.

### UC-04 — Review history and flagged observations

Actor: Authorised user. Requirements: FR-02, FR-07, FR-12, FR-13.

1. Select a time interval and inspect temperature graph and table.

2. Review out-of-range observations with their original profile.

3. If implemented, view estimated excursion periods and record acknowledgement.

Empty and invalid intervals have explanatory states. Gaps are not drawn as proof of continuous safe conditions.

### UC-05 — Recover from a network outage

Actor: Device. Requirements: FR-08, FR-09, FR-10.

1. Continue sampling, local warnings and durable queuing offline.

2. Reconnect and transmit records with unchanged IDs, sequence numbers and times.

3. Delete/mark queue entries delivered only after application acknowledgement of a committed database record; broker PUBACK is insufficient.

4. Show restored contact, while showing the actual age of the latest reading.

A lost acknowledgement causes a safe retry. Records older than the latest sample enrich history without rolling current status back. Exceeding tested capacity or loss of power is reported as a limitation.

### UC-06 — Receive a phone notification

Actor: Authorised user. Requirements: FR-05, FR-14.

1. Enable notifications through the proposed Web Push channel on a supported browser.

2. Receive a Warning or Alert notification containing the box identity, temperature and observation time when connectivity is available.

3. Open the application to check current freshness and history.

Repeated unchanged states must not flood the phone. Denied permission, missing connectivity and delivery failures must be tested. Phone delivery is not guaranteed during an outage; local LED feedback continues.

## 5. Traceability and testing

Detailed preconditions, actions, expected results and an execution record are provided in the Initial Test Plan.

| Use case | Requirements | Planned evidence |
| --- | --- | --- |
| UC-01 | FR-01, FR-02 | Valid/invalid login, logout and denied API access for an unassigned user |
| UC-02 | FR-03, FR-05, FR-06, FR-09, FR-10 | Reference thermometer comparison; synthetic boundary values; disconnected sensor; local LED during Wi-Fi loss |
| UC-03 | FR-02, FR-04, FR-05, FR-08, FR-11 | Fresh, stale, never-connected and read-failure UI demonstrations |
| UC-04 | FR-02, FR-07, FR-12, FR-13 | Filtered history, visible gaps, acknowledgement and delayed-event recalculation |
| UC-05 | FR-08, FR-09, FR-10 | Recorded outage/restart, queue reconciliation and duplicate resend |
| UC-06 | FR-05, FR-14 | Phone receipt, permission denial, repeated-state suppression, delivery failure and delayed-upload scenarios |

Buzzer testing (FR-15), if implemented: local activation on Alert, operation without Wi-Fi and silencing without clearing the warning.

## 6. UX and boundaries

Keyboard-accessible controls, readable labels, Celsius units and accessible table alternative to charts.

Clear observation and receipt times; no colour-only warnings.

No patient records, prescribing, drug-dose accounting or automated medicine-disposal decisions.

One internal temperature sensor only; humidity, weight, external-temperature and opening measurements are outside this release.

The physical LED is a single-colour output device; application status colours do not require an RGB LED.

# Appendix F Initial test plan

# Smart Box / TempSafe - Initial Test Plan

## Purpose

Plan how to verify the functional requirements and selected Universal Design principles. This document describes future testing.

## Equipment and prerequisites

Raspberry Pi 400, Adafruit SCD-41, single-colour LED, suitable resistor, wiring and power supply; prototype box.

Reference thermometer for comparison; record its stated accuracy and placement.

Device software, local SQLite queue, MQTT/TLS broker, ingestion worker, backend, MySQL and web application when available.

Controllable Wi-Fi, a phone with the chosen notification channel and two authorised users assigned to the same physical prototype box, plus an unassigned test user.

Synthetic readings and controlled faults for repeatable boundary/recovery tests; these do not prove physical sensor accuracy.

Browser developer tools, keyboard, contrast checker and a screen reader.

Buzzer and silence control only if implemented. Power off the circuit before changing wiring.

## Settings to record before execution

| Setting | Proposal or required decision |
| --- | --- |
| Sampling interval | 30 seconds |
| Heartbeat interval | 60 seconds |
| Device-contact timeout | 3 minutes |
| Offline capacity | At least 24 hours: 2,880 attempts at 30-second intervals |
| Profile | L < W < U. Demo W = 24°C; U = 25°C. Lower limit L remains to be agreed. |
| Stale-reading threshold | Proposed 90 seconds; agree and test independently of the 180-second contact timeout. |
| Measurement tolerance | Agree from sensor and reference specifications before physical comparison. |
| Notifications | Proposed Web Push; verify browser/platform support, required home-screen installation, permission, authorised recipients, unchanged-state suppression and 60-second online receipt target. |
| LED sensor-error pattern | Proposed two short flashes followed by a pause; agree timings and test recognition. |
| Buzzer, if implemented | Define sound pattern, repetition, silence and reset behaviour. |

## Functional tests

Each case specifies preconditions, actions and expected outcomes. All cases are initially Not Run; collect actual results separately.

| ID / requirements | Preconditions | Actions | Expected result |
| --- | --- | --- | --- |
| TP-01 / FR-01 | User exists; backend reachable. | Sign in correctly; sign out; reuse the old session against a protected endpoint. | Access is granted initially; sign-out invalidates the session and further access is denied. |
| TP-02 / FR-01 | Login available. | Submit wrong credentials and empty fields. | No session is granted; clear errors appear without exposing credentials. |
| TP-03 / FR-02 | One physical box; users A/B assigned to it; user C unassigned. | As A/B, open the shared box; as C, request its data through the UI and API; try a non-existent box ID. | A/B can access the same box; C is denied in both UI and API with a clear no-access state; unknown IDs expose no data. No second physical box is required. |
| TP-04 / FR-03 | Sensor/reference placed together; tolerance agreed. | Compare stable readings under several controlled conditions in the assembled box; assess placement, self-heating and any configured temperature offset; inspect stored records. | Differences meet agreed tolerance; each record has value/unit, ID, box, sequence, time, profile and quality. Record discrepancies. |
| TP-05 / FR-03 | Sampling configured. | Observe multiple attempts; restart normally; inspect subsequent records. | Attempts follow the interval; IDs remain unique and persistent sequence numbers do not cause conflicts after restart. |
| TP-06 / FR-05 | Synthetic input and spaced L/W/U limits available. | Inject each value in the boundary table below, plus invalid data. | Classification matches the table, including equality; invalid values are never Normal. |
| TP-07 / FR-05 | Versioned profiles supported. | Record data, change profile, record again and inspect history; attempt L >= W or W >= U. | Original interpretation remains attached to older observations; invalid profile ordering is rejected. |
| TP-08 / FR-06 | Single-colour LED connected. | Inject Normal, Warning, Alert and Normal in order. | LED is off, on, on and off by the next completed sample; no physical colour change is expected. |
| TP-09 / FR-03–FR-06 | Previously valid Normal and Alert readings available. | Simulate sensor failure in each state; inspect records, LED and dashboard. | Error quality and distinct blinking appear; error is not replaced by zero or Normal; previous valid readings are clearly historical. |
| TP-10 / FR-03, FR-04 | Clock failure can be simulated. | Record with an unreliable clock; inspect display/history. | Clock unreliability is explicit and the reading is not presented as reliably timed fresh data. |
| TP-11 / FR-04, FR-08 | Freshness threshold agreed. | Inspect a box without readings; send data; stop samples but continue heartbeats beyond freshness threshold. | Unknown initially; value/unit, observation and contact times shown separately; contact does not refresh old data. |
| TP-12 / FR-08 | One previously connected and one never-connected device. | Stop contact for more than 3 minutes; restore it. | Not-reporting state and last contact shown; never-connected state distinct; reconnection does not falsely refresh buffered readings. |
| TP-13 / FR-07 | History includes all states, gaps and errors. | Filter by box/time; compare graph/table; try empty/invalid intervals. | Correct records, original times and thresholds appear; gaps/quality issues visible; empty/invalid states explained. |
| TP-14 / FR-06, FR-09 | Device powered; Wi-Fi controllable. | Disconnect Wi-Fi; generate Warning and Alert; inspect LED/queue. | Local sampling, LED and durable queuing continue; phone delivery is not assumed offline. |
| TP-15 / FR-09, FR-10 | Offline capacity environment prepared. | Run offline for 24 hours at 30-second intervals, including a normal restart; reconnect and reconcile IDs/counts. | At least 2,880 attempts supported; queued records survive restart, upload with original IDs/times and show no unexplained loss or duplicates. Accelerated tests supplement the real-duration test. |
| TP-16 / FR-09, FR-10 | Acknowledgement loss can be simulated. | Store an upload but lose acknowledgement; retry identical records; reuse an ID with different content. | Identical retry creates one record; conflicting reuse rejected; unacknowledged data retained. |
| TP-17 / FR-04, FR-10, FR-14 | Newer Normal data current; older Alert data queued. | Upload old records; inspect current state, history and phone. | History gains records without rolling back current state or sending a misleading current Alert. |
| TP-18 / FR-14 | Channel chosen; authorised phone connected and permitted. | Transition Normal → Warning → Alert; record receipt times; open notifications. | Clear box identity, temperature/time delivered within agreed target; link opens correct authorised box. |
| TP-19 / FR-14 | Repeat rules defined. | Send repeated unchanged warnings, then a genuine transition; inspect recipients. | No flood for unchanged states; transitions follow rules; unauthorised recipients receive no box data. |
| TP-20 / FR-14 | Permission/network/provider faults controllable. | Deny permission, disconnect, simulate rejection and restore delivery. | Failures exposed; no false success; LED continues; recovery follows agreed rules without duplicates or misleading old alerts. |

### Boundary expectations for TP-06

| Input | Expected state |
| --- | --- |
| L−0.1 | Alert |
| L, L+0.1 | Normal, provided L+0.1 < W |
| W−0.1 | Normal, provided W−0.1 >= L |
| W, W+0.1 | Warning, provided W+0.1 <= U |
| U−0.1, U | Warning, provided U−0.1 >= W |
| U+0.1 | Alert |
| Invalid / missing | Unknown or sensor error, never Normal |

## Additional secure pub-sub tests

| ID | Actions | Expected result |
| --- | --- | --- |
| SEC-01 | Connect using missing, revoked or untrusted certificates; try plaintext transport. | Connections rejected; no fallback to plaintext. |
| SEC-02 | Publish to another box topic, forge payload box ID or publish an acknowledgement as a device. | Broker ACL or worker rejects the action; no unauthorised records. |
| SEC-03 | Receive broker PUBACK, interrupt ingestion before commit and restart the device. | Pending record remains in SQLite until application acknowledgement after commit. |
| SEC-04 | Replay old heartbeats and upload old samples during an outage. | Contact freshness does not advance without a valid current heartbeat challenge/live sample. |
| SEC-05 | Run cron jobs twice, overlap runs and force backup failure. | No duplicate transition events; locks prevent overlaps; retention does not run without successful backup. |
| SEC-06 | Restore an encrypted backup in an isolated test environment. | Authorised restore succeeds and sample IDs/counts reconcile. |

## Conditional Should Have tests

| ID / requirements | Preconditions | Actions | Expected result |
| --- | --- | --- | --- |
| TP-21 / FR-11 | Automatic refresh implemented. | Upload new data without interacting; interrupt requests. | Dashboard updates within proposed five-second polling interval; refresh failures visible. |
| TP-22 / FR-12 | Acknowledgement implemented. | Review a flagged observation; inspect reviewer/time, reading and warning. | Reviewer/server time recorded; measurement unchanged; active warning not cleared. |
| TP-23 / FR-13 | Summaries implemented. | Submit consecutive out-of-range data, errors, gaps and delayed observations. | Estimated summaries use observation order, break at gaps/errors and recalculate affected history. |
| TP-24 / FR-15 | Buzzer implemented; rules defined. | Trigger Alert online/offline; silence; return to Normal; trigger again. | Local sound follows rules; silence does not clear LED/dashboard; reset/repeat behaviour matches specification. |

## UX and Universal Design tests

| ID / principles and requirements | Preconditions | Actions | Expected result |
| --- | --- | --- | --- |
| UX-01 / P4; FR-04, FR-05, FR-08 | All status screens available. | Hide colour/view greyscale; ask users to identify Normal, Warning, Alert, stale and sensor-error states. | Text/symbols communicate states independently of colour; incorrect interpretations recorded. |
| UX-02 / P4; FR-04, FR-07 | Contrast checker; phone viewport. | Check contrast, 200% zoom, narrow viewport and bright/dim conditions. | Text meets project 4.5:1 contrast target; key values/controls remain readable without clipping; limitations recorded. |
| UX-03 / P3; FR-01, FR-02, FR-04, FR-07 | Task script ready. | Without coaching, ask users to sign in, open the shared assigned box, identify condition and find an earlier flagged reading. | Journey understandable; completion/time/errors recorded and improvements identified. |
| UX-04 / P3/P4; FR-01, FR-02, FR-07 | Keyboard and screen reader available. | Navigate login, the shared box dashboard/history; read status and table with screen reader. | Meaningful labels, visible focus, logical order and no keyboard traps; information accessible without graph. |
| UX-05 / P5; FR-04, FR-08, FR-10 | Fault/stale/delayed scenarios ready. | Ask users whether data is current and what is missing. | Old, missing or invalid data distinguishable; misleading interpretations logged as defects. |
| UX-06 / P4/P5; FR-14 | Test phone notifications available. | Ask users to identify box, temperature/time and next step from Warning/Alert messages. | Messages understandable; link leads to correct box; receipt does not imply a fresh reading. |

## Execution record and evidence

Use Not Run, Blocked, Pass, Fail or Not Applicable. Record reasons for Blocked/Not Applicable. Evidence can include screenshots, paired thermometer readings, queue IDs/counts, API responses, notification timings and participant notes.

| Test ID | Tester / date | Build / settings | Actual result | Status | Evidence / defect link |
| --- | --- | --- | --- | --- | --- |
| To be completed when testing begins | — | — | - | - | — |

# Appendix G Submission review

# IoT G-CA1 submission review

Review date: 9 October 2026.

## Marking scheme coverage

| Criterion | Evidence / remaining action |
| --- | --- |
| Design: title page 2, contents 2, formatting 2 | Design has title metadata, authors and contents links; final exported/printed presentation of the document needs team inspection. |
| Design: references 4, writing 10 | Source list and section references added. Confirm exact earlier morphine/NAS policy/study sources. Proofread final team edits. |
| Hardware: list 2, power 2, Wi-Fi 2, detail 8 | Parts list, pin-level wiring, bench power/Wi-Fi, controller placement, procurement and sensor timing documented. |
| Hardware: Fritzing diagram 6 | Outstanding: SVG/PNG is not a native Fritzing design. Fritzing is not installed on this workstation. Build/open a real .fzz with the correct Pi 400 pin mapping and SCD-41 breakout; export its breadboard/schematic view and verify all nets against prototype-wiring.md. Do not rename SVG to .fzz. |
| Data: gathered 6, storage 6, processing 8 | Contract, 30-second sampling, manufacturer update frequency, SQL schema/queries, SQLite queue and explicit cron entries documented. SQL/jobs are proposals, not executed implementation. |
| Security: device 2, transfer 4, storage 4 | Device credentials/physical controls, MQTT/TLS and HTTPS, authorisation, database and backup controls documented. |
| UI: mock-ups 10, functionality 5, users/testing 5 | Existing presentation concepts extracted into docs/design/ui-design.md; login, access, freshness, errors and history interactions documented. Concepts still need visual updates for missing states; real implementation tests are not required as evidence of an already built application for this design assessment. |
| Git: seven rules 4, frequency 2, contribution 2, review 2 | Genuine incremental commits exist for all three contributors. Review evidence and compliance checks are below; record actual group approval rather than inventing it. |

## Submission requirements

| Item | Status / action |
| --- | --- |
| Repository URL | Included in README and design: https://github.com/nikitsya/Smart-Box. Lecturer access has not been verified. |
| CA cover sheet | Completed IoT CA cover sheet included with the submission. |
| AI cover sheet | No separate completed official AI usage cover sheet located. Obtain and complete the lecturer's form required by the 2026–27 specification. |
| AI log | AI_Chat_Log.md records available use. Add any earlier logs the team possesses and check declaration accuracy. |
| ZIP | Prepared under output/submission/; rebuild after final forms/Fritzing/team edits. Exclude Git, IDE files, caches and credentials. ZIP is a draft, not a submitted assessment. |
| Source files | Include the documentation, assets, presentation and proposed schema. Device/backend/frontend code is not yet implemented; do not claim otherwise. |
| Moodle | Human upload required after checking all critical items and the actual deadline. |

## Genuine contribution evidence

The following commits were observed in local history. They show actual changes, not completion of every assigned task.

| Contributor | Example evidence |
| --- | --- |
| Nikita Smiichyk / nikitsya | 5eeffd5: consolidated design[source], 9437bd9: SQL schema[source]. |
| Hanna Bokariuk / HannaBProjects | ec2eedb: scope[source], cd72000: hardware list[source], 5f99ef0: cover sheet[source]. |
| Maryna Hordiienko | 33468f6: dashboard/website research[source], a661dae: design principles[source]. |

Some historical subjects are vague or contain spelling/capitalisation issues. Do not rewrite shared history merely to improve assessment appearance. For new commits follow the seven rules: separate subject/body with a blank line; keep subject at most 50 characters; capitalise; omit trailing period; use imperative mood; wrap body at 72 characters; explain what/why rather than implementation mechanics.